| 参数名称 (Parameter) | 单位 (Unit) | 配置 1 (24V) | 配置 2 (48V) | 配置 3 (48V) |
| --- | --- | --- | --- | --- |
| 额定电压(Rated voltage) | V | 24 | 48 | 48 |
| 电压范围(Voltage Range) | V | 12~56 | 12~48 | 12~48 |
| 额定功率(Rated Power) | W | 252 | 134 | 182 |
| 额定扭矩(Rated torque) | N.M | 5.00 | 4.60 | 5.40 |
| 堵转扭矩(Peak torque) | N.M | 11.00 | 17.91 | 17.21 |
| 减速后额定转速(Rated Speed after reduce) | RPM | 120 | 200 | 255 |
| 减速后最大转速(Max Speed after reduce) | RPM | 420 | 280 | 322 |
| 额定电流(Rated current) | A | 10.5 | 2.8 | 3.8 |
| 堵转电流(Peak current) | A | 23.40 | 17.19 | 19.50 |
| 相间电阻(Phase-to-phase resistance) | Ω | 0.44 | 1.45 | 1.45 |
| 相间电感(Phase-to-phase inductance) | mH | 0.23 | 0.823 | 0.82 |
| 转速常数(Speed constant) | rpm/v | 11.54 | 5.83 | 6.71 |
| 扭矩常数(Torque constant) | N.M/A | 0.47 | 1.20 | 1.20 |
| 极对数(Number of pole pairs) | Pairs | 14 | 14 | 14 |
| 减速比(Gear Rate) | / | 8:1 | 8:1 | 8:1 |
| 减速器类型(Gear type) | / | Planetary | Planetary | Planetary |
| 减速器齿轮材料(Reducer gear material) | / | STEEL | STEEL | STEEL |
| 减速器齿轮背隙(Reducer gear backlash) | arcmin | 15 | 15 | 15 |
| 电机重量不带驱动(Motor weight without driver) | g | 370 | 370 | 370 |
| 电机尺寸不带驱动(Size without Driver) | mm | Ø80*29.5 | Ø80*29.5 | Ø80*29.5 |
| 轴向最大负载(Max.Axial load) | N | 200 | 200 | 200 |
| 径向最大负载(Max.Radial load) | N | 800 | 800 | 800 |
| 噪音(Noise) | dB | <60 | <60 | <60 |
| 是否支持第二编码器 (the second encoder on the output shaft) | / | YES | YES | YES |
| 防水防尘等级(Protection grade) | / | IP54 | IP54 | IP54 |
| 工作温度范围(Working temperature) | °C | -20~+80 | -20~+80 | -20~+80 |